Assessment of bone quality in metabolic bone diseases

Scientific problem and socio-economic reasons:

About 30% of all postmenopausal women in the European Union have osteoporosis, and the prevalence continues to increase with the progressively aging population. The resulting pain and loss of function has a significant negative effect on the quality of life. Hence, effective diagnostics, prevention and treatment are essential for successful management of osteoporosis.

Objectives and methods:

This project derived quantitative interrelationships between parameters that reflect bone quality in bone from human cadavers with respect to age and different metabolic bone diseases. The main focus was on osteoporosis. In addition to measurement of bone mineral density (BMD), which is the current diagnostics method, several factors related to bone quality, i.e. bone architecture (microCT), geometry (Xray and CT), turnover (histomorphometry), composition (spectroscopy) and mechanical strength (nanoindentation, finite element analysis), that contribute significantly to fracture risk were evaluated in this project. Also, novel backscattered ultrasound methods are used for trabecular bone diagnostics and to assess their feasibility for clinical practice.

The aims were to improve the understanding of bone quality in health and disease, with the long term goal to more effectively diagnose early osteoporosis and predict more accurately fracture risk. Specifically, we aim to assess the effects of aging and osteoporosis on structural, compositional and mechanical properties of trabecular bone. To reach our aims, human cadaver material was supplemented with bone biopsies (iliac crest) from clinical patients with different metabolic bone diseases. Additionally, methodological studies to determine relationships between biophysical techniques were conducted with animal bone tissue.

Main findings:

# Assessment of the 3-D shape of the proximal femur using a shape template and a bone mineral density image: Measurement of BMD by DXA (Dual energy X-ray absorptiometry) alone is only a moderate predictor of fracture risk. Finite element analyses (FEA) of bone strength can contribute to a more accurate prediction of fracture risk. A method was developed to estimate the 3D geometrical shape of bone based on a 2D BMD image and a femur shape template (derived from CT images). The volumetric errors resulting from the shape estimation were low. However, small geometrical errors led systematically to somewhat larger errors in the evaluation of bone strength using FEA. With further development, this method could provide invaluable information on mechanical strength, and e.g. when coupled with dual energy X-ray absorptiometry (DXA) measurements, could ultimately lead to more sensitive diagnosis of osteoporosis.

MicroCT image resolution affects structural parameters of normal and osteoporotic human trabecular bone differently: Based on bone biopsies of clinical patients and cadavers, we assessed how microCT image resolution affects structural parameters of bone. We hypothesized that image resolution affects bone structural parameters differently in healthy and osteoporotic trabecular bone. Bone samples were imaged with a high-resolution microCT (16-18 µm voxels). The effect of decreasing image resolution was compared between the normal and osteoporotic samples. The results suggest that structural differences between osteoporotic and normal trabecular bone may not be reliably detected with CT scanners providing image voxel sizes above 100µm.

Metabolic status of human trabecular bone affects the reproducibility and agreement of MicroCT and histomorphometry: This study determined whether the reproducibility and agreement of MicroCT and histomorphometry depend on the metabolic status of bone. Iliac crest biopsies were obtained from the healthy subjects and from patients with osteoporosis and renal osteodystrophy. The differences between disease groups were generally more profound with histomorphometry than with microCT. The reproducibility seems to be better for bone with increased metabolism and turnover, and poorer in low turnover bone. This may be explained by thinner trabeculae, lower amount of osteoid and lower number of bone cells.

Infrared spectroscopy indicated altered bone turnover and remodelling activity in osteoporosis and renal osteodystrophy: Osteoporosis and renal osteodystrophy (ROD) alter metabolic activity and remodeling rate of bone, and may lead to different bone composition. This study characterized the composition of bone in these diseases by means of Fourier transform infrared spectroscopic imaging (FTIRI). Iliac crest biopsies from healthy bone and patients with renal osteodystrophy were analyzed. The ROD samples showed lower carbonate content and a lower degree of mineralization at the edges of the trabeculae. Correlations with histomorphomety suggested that FTIRI parameters of carbonate may be indicative of turnover and remodeling rate of bone. These findings are potentially interesting for future diagnostics applications.

Cortical bone tissue increases its elastic stiffness but becomes less viscoelastic during early age: It has been proposed that especially changes in the collagen phase of bone are responsible for making the bone more fragile, and potentially less viscoelastic. In these studies, nanoindentation was applied to determine hardness and reduced modulus (elastic), as well as creep, and semi-dynamic testing (viscoelastic), to quantify changes in viscoelastic properties with age. Cortical bone samples from female rabbits of varying age were examined. The elastic stiffness of bone increased significantly with age, whereas the viscoelastic parameters decreased with age. This indicated significant structural and functional maturation of the matrix with age.

Numerical analysis of uncertainties in dual frequency bone ultrasound technique: Quantitative ultrasound (QUS) is a potential future diagnostics technique for osteoporosis. However, variation in the overlying soft tissue causes problems in vivo. This study addressed the elimination of these errors using the dual frequency ultrasound (DFUS) technique numerically. The significant soft tissue induced errors in integrated reflection coefficient values (>300 %) could be effectively minimized (<10 %) by means of the DFUS correction. Importantly, after the DFUS correction physiologically relevant variation in the cortical bone density could be detected.

Conclusions and potential socioeconomic impact:

- The developed method to assess 3D bone strength based on a 2D image, a shape template, and FEA, could ultimately lead to more sensitive assessment of fracture risk in osteoporosis.

- Trabecular structural parameters in normal and osteoporotic bone can only be separated with image voxel sizes up to 100µm. This has clinical importance for development and application of quantitative imaging tools.

- Spectroscopic identification of differences in bone composition between patients with osteoporosis and renal osteodystrophy suggests this method to be a future potential diagnostic method for metabolic bone diseases.

- Methods to evaluate viscoelastic properties of bone were developed and it was shown that viscoelasticity decreases with age. This may explain increased bone fragility in elderly.